物质安全资料表 MSDS

版本Version ：2.1

1. 产品与厂商资料 / Product and Company Identification

| 产品名称 ： | 水性环氧绝缘树脂 |
| --- | --- |
| Product name: | Waterborne epoxy insulating resin |
| 产品型号Product model ： | FS7100-115M · |
| 制造商名称 ： | 广州飞思合成材料有限公司 |
| Manufacturer: | Guangzhou Faith Synthetic Materials Co., Ltd. |
| 制造商地址 ： | 广东省广州市黄埔区永和经济开发区沧海2路3号 |
| Manufacturer address: | No. 3, Canghai 2nd Road, Yonghe Economic Development Zone, Huangpu District, Guangzhou, Guangdong, China |
| 紧急联络电话 Emergency telephone： | +86-020-82793066 · |
| 传 真Fax ： | +86-020-32636951 · |

2. 成分辨识资料 / Composition / Information on Ingredients

| 序号 No. | 成分名称 Ingredient | 含量 Content | 分子式 Molecular formula | CAS No. |
| --- | --- | --- | --- | --- |
| 1 | 环氧树脂 Epoxy resin | 10-15% | [C11H12O3]n | 61788-97-4 |
| 2 | 消泡剂 Defoamer | 0.25-0.5% | C3H9OSi.(C2H6OSi)n.C3H9Si | 9006-65-9 |
| 3 | 乳化剂 Emulsifier | 2-3% | [C11H12O3]n O(CH₂CH₂O)nH | -- |
| 4 | 固化剂 Curing agent | 1-1.5% | C8H16N2O4S | 342573-75-5 |
| 5 | 水 Water | 80-85% | H2O | 7732-18-5 |

3. 危害辨识资料 / Hazards Identification

| 健 康 危 害： | 无。 |
| --- | --- |
| Health hazards: | None. |
| 物理性及化学危害： | 无。 |
| Physical and chemical hazards: | None. |
| 危险性类别： | 非危险品。 |
| Hazard category: | Not classified as dangerous goods. |

4. 急救措施 / First-Aid Measures

| 皮 肤 接 触： | 以肥皂水和清水彻底冲洗受污部位5分钟或直到污染物除去。 |
| --- | --- |
| Skin contact: | Wash the contaminated area thoroughly with soap and water for 5 minutes or until the contaminant is removed. |
| 眼 睛 接 触： | 1.提起眼睑，用流动清水或生理盐水冲洗眼睛20分钟或直到污染物除去。 |
| Eye contact: | 1. Lift the eyelids and rinse eyes with running water or saline for 20 minutes or until the contaminant is removed. |
|  | 2.立即就医。 |
|  | 2. Seek medical attention immediately. |
| 食 入 ： | 饮足量温水，催吐。就医。 |
| Ingestion: | Drink sufficient warm water, induce vomiting and seek medical attention. |
| 吸 入 ： | 1.脱离现场至空气新鲜处。 |
| Inhalation: | 1. Remove the person from the site to fresh air. |
|  | 2.若有不适症状立即就医。 |
|  | 2. Seek medical attention immediately if discomfort occurs. |

5. 灭火措施 / Fire-Fighting Measures

| 燃 烧 产 物 ： | 一氧化碳、二氧化碳。 |
| --- | --- |
| Combustion products: | Carbon monoxide and carbon dioxide. |
| 适用灭火剂 ： | 雾状水、二氧化碳、泡沫、化学干粉，消防沙。 |
| Suitable extinguishing media: | Water mist, carbon dioxide, foam, dry chemical powder and fire sand. |
| 消 防 作 业 ： | 疏散人员并隔离火场区域，火灾发生场所严禁非相关人员进入，消防救护时必需配备特殊防护设备，可能的话将容器从火场移至空旷处。灭火时必需位于安全位置或安全距离，用水雾灭火可能无效。在火熄灭后且确保不再复燃的情况下，可用水喷雾来冷却火场和产品容器。 |
| Fire-fighting procedures: | Evacuate personnel and isolate the fire area. Unauthorized personnel are strictly prohibited from entering the fire site. Special protective equipment must be used during fire-fighting and rescue operations. If possible, move containers from the fire area to an open area. Fight the fire from a safe position or safe distance; water mist may be ineffective. After the fire is extinguished and reignition is not expected, water spray may be used to cool the fire area and product containers. |
| 消防防护装备： | 消防人员必须穿着消防衣，配戴防护手套、防护眼镜、防护鞋及呼吸用自给式保护器具。 |
| Fire-fighting protective equipment: | Firefighters must wear fire-fighting clothing, protective gloves, protective goggles, protective footwear and self-contained breathing apparatus. |

6. 意外泄漏处理方法 / Accidental Release Measures

| 注 意 事 项 ： | 迅速撤离泄漏污染区人员至安全区，并进行隔离，禁止非相关人员进入，由专业人员负责清理，建议应急处理人员戴自给正压式呼吸器，穿一般作业工作服。使用适当防护装备清除泄漏。 |
| --- | --- |
| Precautions: | Evacuate personnel from the contaminated leakage area to a safe area promptly and isolate the area. Unauthorized personnel are prohibited from entering. Clean-up shall be performed by qualified personnel. Emergency responders are advised to wear positive-pressure self-contained breathing apparatus and general work clothing. Use appropriate protective equipment to remove the leakage. |
| 环境注意事项 ： | 避免污染水道、饮用水源、地面水和土壤。 |
| Environmental precautions: | Avoid contamination of waterways, drinking-water sources, surface water and soil. |
| 清 理 方 法 ： | 少量泄露：用沙围起隔离，用沙、布料等物质吸收处理，收集至有标识之容器中。大量泄漏：构筑围堤或挖坑收容。用泡沫覆盖，降低蒸气灾害，联络消防队、紧急处理单位及供应商以寻求协助。 |
| Clean-up methods: | Small leakage: contain with sand, absorb with sand, cloth or similar materials, and collect into labeled containers. Large leakage: build dikes or dig pits for containment. Cover with foam to reduce vapor hazards, and contact the fire department, emergency response unit and supplier for assistance. |

7. 操作与储存方法 / Handling and Storage

| 操作注意事项： | 操作时在通风良好场所，使用防护性装备，如防护手套，防护眼镜等，避免皮肤，眼睛直接接触，并且避免发生泄漏。 |
| --- | --- |
| Handling precautions: | Handle in a well-ventilated area. Use protective equipment such as protective gloves and goggles. Avoid direct contact with skin and eyes and prevent leakage. |
| 储存注意事项： | 避免阳光直接照射，避免受热，远离电源、火源、热源，保存在干燥阴凉通风处，容器保持良好密闭性。 |
| Storage precautions: | Avoid direct sunlight and heat. Keep away from power sources, ignition sources and heat sources. Store in a dry, cool and ventilated place, and keep containers tightly closed. |

8. 接触控制/个体防护 / Exposure Controls / Personal Protection

| 工 程 控 制 ： | 提供良好的自然通风条件。 |
| --- | --- |
| Engineering controls: | Provide good natural ventilation. |
| 个人防护设备： | 保护性防渗手套，避免皮肤接触。 |
| Personal protective equipment: | Protective impermeable gloves; avoid skin contact. |
| 呼 吸 防 护 ： | 无需特殊防护。 |
| Respiratory protection: | No special protection is required. |
| 眼 睛 防 护 ： | 建议使用化学防溅护目镜、面罩。 |
| Eye protection: | Chemical splash goggles and a face shield are recommended. |
| 身 体 防 护 ： | 使用能阻挡本产品的保护衣物，视操作情况使用防护靴子、防护手套、防护衣服等物，洗眼器和紧急淋浴设备。 |
| Body protection: | Use protective clothing capable of blocking this product. Depending on the operation, use protective boots, gloves, clothing and other protective articles, as well as eyewash and emergency shower equipment. |
| 卫 生 护 理 ： | 远离食物、饮水、动物饲料，操作后及休息前洗手 |
| Hygiene measures: | Keep away from food, drinking water and animal feed. Wash hands after handling and before breaks. |
| 其 他 防 护 ： | 工作现场严禁吸烟。保持良好的卫生习惯。 |
| Other protection: | Smoking is strictly prohibited at the workplace. Maintain good hygiene practices. |

9. 物理及化学性质 / Physical and Chemical Properties

| 外观及性状： | 白色乳液 |
| --- | --- |
| Appearance and properties: | white emulsion |
| 气味 ： | NA |
| Odor: | NA |
| 蒸气压 ： | ﹤21.07mmHg （23℃） |
| Vapor pressure: | ﹤21.07mmHg （23℃） |
| 蒸气密度 ： | 无相关数据 |
| Vapor density: | No relevant data |
| 沸点 ： | 无相关数据 |
| Boiling point: | No relevant data |
| 爆炸下限 ： | 无相关数据 |
| Lower explosive limit: | No relevant data |
| 爆炸上限 ： | 无相关数据 |
| Upper explosive limit: | No relevant data |
| 溶解度 ： | 无相关数据 |
| Solubility: | No relevant data |
| 密 度 ： | 1.0～1.05g/cm3（20℃） |
| Density: | 1.0～1.05g/cm3（20℃） |

10. 稳定性及反应活性 / Stability and Reactivity

| 化学稳定性 ： | 正常状况下稳定。 |
| --- | --- |
| Chemical stability: | Stable under normal conditions. |
| 应避免状况 ： | 长期暴露受热。 |
| Conditions to avoid: | Avoid prolonged exposure to heat. |
| 危 险 反 应 ： | 无 |
| Hazardous reactions: | None |

11. 毒性资料 / Toxicological Information

急性毒性 / Acute toxicity:

| 危害物质 Hazardous substance |  | LD50 |  | LC50 |
| --- | --- | --- | --- | --- |
| 环氧树脂 Epoxy resin |  | > 10,000mg/kg（大鼠、吞食） > 10,000mg/kg (rat, oral) |  | -- -- |
| 慢 性 毒 性 ： |  | 皮肤接触可能会引起过敏，重复暴露于树脂中不会引起明显的不良反应。 |  |  |
| Chronic toxicity: |  | Skin contact may cause sensitization. Repeated exposure to the resin is not expected to cause obvious adverse reactions. |  |  |

12. 生态资料 / Ecological Information

| 生态毒性 ： | 可能会对水生环境造成长期的不利影响。 |
| --- | --- |
| Ecotoxicity: | May cause long-term adverse effects to the aquatic environment. |
| 生物降解性 ： | 环氧树脂不易生物分解，但在环境条件下并非无法分解。 |
| Biodegradability: | Epoxy resin is not readily biodegradable, but it may degrade under environmental conditions. |
| 非生物降解性： | 无相关数据。 |
| Abiotic degradation: | No relevant data. |

13. 废弃处置方法 / Disposal Considerations

| 废弃物性质 ： | 危险废物。 |
| --- | --- |
| Waste classification: | Hazardous waste. |
| 废弃处置方法： | 所有废弃及清理方法（填埋或焚化）需遵循国家、省、市、地方的相关法规。 |
| Disposal method: | All disposal and clean-up methods, including landfill or incineration, shall comply with applicable national, provincial, municipal and local regulations. |
| 废弃注意事项： | 严禁倾入下水道或地表水，严禁随意放置，粘有树脂的所有容器或吸收物应按危险废物处理。 |
| Disposal precautions: | Do not discharge into sewers or surface water. Do not place waste casually. All resin-contaminated containers or absorbents shall be handled as hazardous waste. |

14. 储存、运输资料 / Storage and Transport Information

| 危险货物编号 ： | 无 |
| --- | --- |
| Dangerous goods number: | None |
| UN 编号 ： | 无 |
| UN number: | None |
| 储存注意事项 ： | 最佳储存温度为15～30℃，并存放于干燥通风处所，需密封保存，防止各种湿度造成的影响。 |
| Storage precautions: | The optimum storage temperature is 15-30°C. Store in a dry, ventilated place and keep sealed to prevent effects caused by humidity. |
| 运输注意事项 ： | 运输时运输车辆应配备相应品种和数量的消防器材及泄漏应急处理设备。夏季最好早晚运输。运输时避免掉落、倾倒、冲撞，运输途中应防曝晒、雨淋，防高温。中途停留时应远离火种、热源、高温区。装填于密闭容器内运送，运输需遵守国家及国际间运输相关法规。 |
| Transport precautions: | During transport, vehicles shall be equipped with appropriate types and quantities of fire-fighting equipment and leakage emergency handling equipment. In summer, transport is preferably arranged in the morning or evening. Avoid dropping, tipping and impact during transport, and protect from direct sunlight, rain and high temperature. During stops, keep away from ignition sources, heat sources and high-temperature areas. Transport in closed containers and comply with national and international transport regulations. |

15. 法规资料 / Regulatory Information

| 法 规 信 息 ： | 生产、储存、运输及废弃处理需遵循当地的国家、省、市、地方的相关法规。 |
| --- | --- |
| Regulatory information: | Production, storage, transport and disposal shall comply with applicable national, provincial, municipal and local regulations. |

16. 其他资料 / Other Information

| 以上记载内容请询问： |  |
| --- | --- |
| For inquiries regarding the above information, please contact: |  |

广州飞思合成材料有限公司 技术部
Technical Department, Guangzhou Faith Synthetic Materials Co., Ltd.

记载内容是基于收集可能的资料、资讯、数据等形成，其物理化学性质的数据并非保证值，实际上使用者要与当地的国家、地方法规结合运用。
The information recorded herein is based on available materials, information and data collected. The physicochemical property data are not guaranteed values. Users shall apply this information in combination with applicable national and local regulations.

修订日期Revision Date：03/2026